ANNEX II

TECHNICAL SPECIFICATIONS AND MAX AVAILABLE PRICE PER ITEM

ANNEX 2: Technical specifications

| Item | Technical specification | Quantity | Max Unit price (€) | Max Total price (€) |
| --- | --- | --- | --- | --- |
| Weather station |  | 1 | 6500 | 6500 |
| - Air Temperature sensor | Measuring range - 20 to 60 °C Accuracy: < 0.5 °C | 1 |  |  |
| - Air Relative Humidity sensor | Measuring range Accuracy : 5% | 1 |  |  |
| - Solar radiation sensor | Measuring range 0 .to 1500 W/m² spectral range 290 to 3 000 nm Accuracy : 5% | 1 |  |  |
| - Wind speed sensor | Measuring range 0 .to 40 m/s Accuracy : 5% | 1 |  |  |
| - Rainfall sensor | Funnel Area : ≥ 200 cm2 Accuracy : 5% With protection from bird | 1 |  |  |
| - Accessories | This station must be equipped with mat(s), anchoring system and all accessories for fixing the various sensors and the power system. | 1 |  |  |
| Soil moisture and temperature multilayers probe |  | 24 | 990 | 23760 |
|  | Measuring range : 0 – 0.80 Vwater/Vsoil Minimum 6 depth : 0 to 60 cm Accuracy : 0.01 | 19 |  |  |
|  | Measuring range : 0 – 0.80 Vwater/Vsoil Minimum 6 depth : 0 to 80 cm Accuracy : 0.01 | 5 |  |  |
| Pulse water flowmeter |  | 12 | 450 | 5400 |
|  | Diameter range : DN40 to DN 50 Measuring range 0 to 25 m3/h (nominal) Operating Pressure: 0.5 - 15 Bar Accuracy : 5% | 12 |  |  |
| Solenoid valve for irrigation water |  | 12 | 170 | 2040 |
|  | Diameter range: DN40 to DN 50 Operating Pressure: 0.5 - 10 Bar | 12 |  |  |
| Gateways |  | 15 | 900 | 13500 |
| - Indoor Gateways | 1 gateway on-site 1 gateway can support 4-8 sensors Compliance to LoRaWAN | 13 |  |  |
| - Outdoor Gateways |  | 2 |  |  |
| IoT Platform (for reading, viewing, downloading, configuring data upload, etc.) |  | 1 | 1550 | 1550 |
|  | All fields Sensor data (sensors Network) to be synchronized with the IoT platform Fields station/Sensors configuration Maps & Geolocation Services Data Processing & Alerting Dashboards & Reports Open API for Easy Integration | 1 |  |  |
| Installing of the equipment |  |  | 2250 | 2250 |

Additional Specifications:

- All sensors must have an operating temperature range between -20 ºC and +60 ºC.

- Energy supply for each equipment must be done by battery or solar panel

- The sensors / devices must be equipped with all necessary accessories for installation and operation: protective case, mat(s), support(s), an anchoring system, etc.

- All sensor/device enclosures and accessories must be resistant to water, dust and radiations.

- Alert system regarding sensor/devices malfunction is required.

- Access to raw sensor data (in direct or indirect form) is required for validation of field data.

- All sensors have to be connected and operate on LoRaWAN® platform with a connectivity of 1 year.

- Within the 1 year following the contract signature, the connection of all sensors will be transferred to a Local LoRaWAN®. To this end, the company is requested to provide technical support to facilitate the transfer.